FLAIRE: AI Offline-First Flood Early Warning System Using LoRa Sensors for Brgy. Elvita, Narra, Palawan

An Undergraduate Thesis Proposal

Presented to the faculty of the

Occidental Cluster-Narra Campus

Palawan State University

In Partial Fulfillment of the

Requirements for the Degree

Of Bachelor of Science in Computer Science

By

Betita, Nathaniel S.

January 2026

APPROVAL SHEET

This thesis is entitled “FLAIRE: AI OFFLINE-FIRST FLOOD EARLY WARNING SYSTEM USING LORA SENSORS FOR BRGY. ELVITA, NARRA, PALAWAN” prepared and submitted by Betita, Nathaniel S. in partial fulfilment of the requirements for the degree of Bachelor of Science in Computer Science, is hereby accepted.

JOEL P. GULANE JR., MIT

Research Adviser

PANEL OF EXAMINERS

Approved by the Panel on Oral Thesis/ Defense Examination with a grade of _____________.

JUAN MIGUEL B. RODRIGUEZ

Chairperson

EARL ASI M. MARTINEZ                                                 ERROL A. FUERTES

Member                                                                                 Member

Accepted and approved in partial fulfilment for the degree of Bachelor of Science in Computer Science.

REMEDIOS L. MAGBANUA, MAEd

Campus Director

Date: _______________

ACKNOWLEDGEMENT

Abstract

Betita, Nathaniel S. “FLAIRE: AI OFFLINE-FIRST FLOOD EARLY WARNING SYSTEM USING LORA SENSORS FOR BRGY. ELVITA, NARRA, PALAWAN”

Table of Contents

Page

Approval Sheet	ii

Acknowledgement	iii

Abstract	iv

Table of Contents	v

List of Tables	vii

List of Figures	viii

List of Abbreviation and/or Symbols	ix

Chapter 1: Introduction	1

Background of the Study	1

Statement of the Problem	4

Significance of the study	5

Scope and Delimitations	6

Definition of Terms	8

Chapter 2: Theoretical Framework	10

Review of Related Literature	10

Review of Related Studies	17

Conceptual Framework	24

Research Paradigm	25

Chapter 3: Research Methodology	27

Research Design	27

Locale of the Study	28

Flowchart of the Proposed System	29

Target Respondents	34

Instrumentation	34

Gantt Chart	35

Cost and Benefits Analysis	36

System Maintenance Plan	39

Developer and Client Communication	39

Chapter 4: Results And Discussion	41

Chapter 5: Summary Of Findings, Conclusions, And Recommendations	42

References	43

Bionotes	47

Appendices	48

List of Tables

Page

Table 3.1 Cost Analysis36

List of Figures

Page

Figure 2.1. Schematic presentation of conceptual framework 27

Figure 3.1. DRM Framework28

Figure 3.2. Locale of the Study30

Figure 3.3.1. LoRa-enabled Hydrological sensor nodes flowchart31

Figure 3.3.2. Edge device flowchart32

Figure 3.3.3. LAN Dashboard34

Figure 3.4. Gantt Chart37

List of Abbreviation and/or Symbols

| AI | Artificial Intelligence |
| --- | --- |
| ANN | Artificial Neural Network |
| API | Application Programming Interface |
| ASTI BDRRMC | Advanced Science and Technology Institute Barangay Disaster Risk Reduction and Management Committee |
| BT | Bluetooth |
| CSU-MLP | Colorado State University Machine Learning Probabilities system |
| DIMER | Decentralized Intelligent Model Exchange Repository |
| DNN | Deep Neural Network |
| DOST | Department of Science and Technology |
| DRM DRRM | Design Research Methodology Disaster Risk Reduction Management |
| ESP32 | Espressif Systems 32-bit microcontroller |
| EW4A | Early Warnings for All |
| EWS4 | Flood Forecasting and Early Warning System for the Greater Metro Manila and Laguna Lake Complex Project. |
| FFEWS | Flood Forecasting and Early Warning System |
| FLAIRE | Flood Level AI Risk Evaluation |
| GRU | Gated Recurrent Unit |
| GSM | Global System for Mobile communications |
| HEC‑HMS | Hydrologic Engineering Center – Hydrologic Modeling System |
| ICT | Information and Communications Technology |
| IoT | Internet of Things |
| IPO | Input-Process-Output |
| ITU | International Telecommunication Union |
| KoICA | Korea International Cooperation Agency |
| LAN | Local Area Network |
| LoRa | Long Range |
| LoRaWAN | Long Range Wide Area Network |
| LSTM | Long Short-Term Memory |
| MDRRMO | Municipal Disaster Risk Reduction Management Office |
| MHEWS | Multi-Hazard Early Warning Systems |
| ML | Machine Learning |
| NDRRMP | National Disaster Risk Reduction and Management Plan |
| NRMSE | Normalized Root Mean Square Error |
| NSE | Nash-Sutcliffe Efficiency |
| PAGASA | Philippine Atmospheric, Geophysical and Astronomical Services Administration |
| PBDHM | Physically‑Based Distributed Hydrological Model |
| PBIAS | Percentage Bias |
| PIN | People in Need |
| PSU | Palawan State University |
| RBMES | Results‑Based Monitoring and Evaluation System |
| R&D | Research and Development |
| SMS | Short Message Service |
| SOM | Self‑Organizing Map |
| SVM | Support Vector Machine |
| TC | Tropical Cyclone |
| UN | United Nations |
| WRF | Weather Research and Forecasting Model |

Chapter 1

INTRODUCTION

Background of the Study

Flooding is one of the most common and devastating natural disasters worldwide, affecting millions of people annually and causing significant economic losses, infrastructure damage, and loss of life. In the Philippines, flooding poses a persistent threat due to the country's geographic location, tropical climate, and seasonal monsoon patterns. Rural communities, particularly those situated along riverbanks, are especially vulnerable to riverine flooding, which occurs when rivers overflow their banks due to excessive rainfall, upstream runoff, or prolonged precipitation.

Brgy. Elvita, located in the municipality of Narra, Palawan, is a flood-prone community that experiences seasonal riverine flooding during the monsoon and typhoon seasons. The barangay has been identified by the Provincial Disaster Risk Reduction and Management Council as one of the more flood-vulnerable areas in Palawan, which contributed to its selection as a pilot site for a provincial Flood Early Warning System (FEWS) installation (Radyo Natin Nationwide, 2020). Provincial and local reports indicate that Barangay Elvita and adjacent communities have repeatedly experienced flash floods that inundate agricultural lands for extended periods, causing economic disruption and placing lives and property at risk. The barangay's proximity to a major river system, combined with limited structural flood-mitigation works, unreliable internet connectivity, and continued reliance on threshold-based warning practices, exposes residents to persistent flood risk. Despite these efforts, the system's reactive nature, dependent on real-time thresholds and online access, limits predictive lead time during network outages common in typhoons. In many rural settings such as Brgy. Elvita, traditional flood warning approaches depend largely on manual river observation, water-level thresholds, and reactive alert mechanisms, which may not consistently provide sufficient lead time for effective evacuation and preparedness.

Current flood warning approaches in many Philippine barangays face several important limitations. Numerous rural communities depend on manual monitoring by Barangay Disaster Risk Reduction and Management Committee (BDRRMC), which can be labor-intensive and prone to delays or human error. Traditional threshold-based systems often trigger alerts only when water levels exceed predetermined limits, with limited consideration of forecast rainfall or the rate of water level increase. In addition, Existing Flood Early Warning Systems deployed by the Department of Science and Technology – Advanced Science and Technology Institute (DOST-ASTI) transmit sensor data to centralized servers, where it is accessed by DRRM personnel through web-based platforms. However, access to these systems may be limited during network outages common in rural areas during typhoons. As a result, many existing systems provide limited predictive capability for anticipating flood risk hours or days in advance, reducing the lead time available for communities to prepare and respond. This underscores the need for offline predictive enhancements such as AI-based flood risk classification, supported by local hydrological data for improved decision-making.

Recent advances in Internet of Things (IoT) technologies, wireless sensor networks, and artificial intelligence offer promising opportunities to address these limitations. LoRa (Long Range) wireless communication technology supports the deployment of low‑power device that can transmit real-time water-level data from remote river locations to a local edge device without requiring internet connectivity. Cached weather forecast data, pre-downloaded daily when internet is available, can provide rainfall predictions, atmospheric pressure, humidity, and precipitation probability integrated with ground‑based sensor observations stored in a local database. Machine learning algorithms that can run on local edge hardware to analyze these multi‑source data inputs and estimate flood risk with improved lead times and potentially higher predictive performance compared to purely threshold‑based approaches.

Despite these technological advancements and the presence of national and regional flood forecasting initiatives in the Philippines, there are still relatively few documented implementations that combine cached weather forecast data, real-time river sensor measurements, AI-based analytics running on local hardware, combined with multi-channel alert dissemination, within a single modular offline-first architecture tailored for rural barangay-level deployment. Moreover, there is limited empirical research at the barangay scale that systematically evaluates and compares offline AI-based flood risk classification systems with traditional threshold-based warning methods in the Philippine context.

This study will address these gaps by designing and developing FLAIRE (Flood Level AI Risk Evaluation), an offline-first AI-based riverine flood risk classification and multi-channel early warning system tailored for Brgy. Elvita, Narra, Palawan. The FLAIRE system will collect real-time water level measurements from 2 LoRa-based hydrological sensors deployed along the river and transmits them to a local edge device via LoRa (primary) and SMS fallback for redundancy. At the edge device, a Machine Learning classification model will analyze the sensor data alongside cached 7-day weather forecasts to generate flood risk classifications levels which are Receding, Alert, Alarm, Critical, providing a 24–48-hour predictive decision window. BDRRMC personnel will be able to access real-time data, AI predictions, and visualizations through a local web dashboard accessible via LAN with no internet required. Personnel will make the final call to activate sirens, queued SMS, or social media alerts, ensuring human oversight while benefiting from AI-enhanced predictive lead time over purely reactive threshold-based methods. This hybrid approach will address limitations of current flood warning systems, where sensor data is SMS-sent to central servers for manual review via websites, by enabling offline local analysis and decision-making during network outages common in typhoons. The model will utilize historical meteorological data from the Philippine Atmospheric, Geophysical and Astronomical Services Administration (PAGASA), local flood records, and real-time sensor data collected during deployment.

By combining real-time sensor data, cached predictive weather information, locally-run machine learning classification, and offline-capable multi-channel alert delivery, this study aims to demonstrate a practical, scalable, and internet-independent approach to barangay-level flood early warning that can be scientifically evaluated and, if proven effective, potentially adapted for other flood-prone communities across the Philippines.

Statement of the Problem

Specifically, this study will answer the following questions:

- What are the key problems and limitations in current flood warning practices and data utilization in Brgy. Elvita, Narra, Palawan, and how can they be addressed by an AI-based system?

- How can an AI-based riverine flood risk prediction system be designed and developed for Brgy. Elvita, integrating:

  - Cached weather forecast data updated daily when internet connectivity is available,

  - LoRa-based hydrological sensor data

  - A Machine Learning model for classifying flood risk levels (Receding, Alert, Alarm, Critical) within a 24–48-hour predictive decision window, and

  - A local web dashboard for BDRRMC personnel to monitor real-time data and model outputs?

- How does the AI-based flood risk classification model compare with a traditional threshold-based warning method using confusion matrix analysis and performance metrics such as accuracy, precision, recall, F1-score, false alarm rate, and lead time?

- How acceptable is the developed system to target users and stakeholders in terms of functionality, usability, reliability, and perceived improvement in disaster preparedness?

Significance of the study

This study will provide practical value to the Barangay Disaster Risk Reduction and Management Committee (BDRRMC) of Brgy. Elvita by delivering an offline-first, AI-enhanced flood early warning system with a 24–48-hour predictive decision window. Through a LAN-accessible web dashboard, the system will support informed decision-making, preemptive evacuation planning, and resource allocation. Barangay officials and residents will benefit from timely and redundant flood notifications via SMS, improving preparedness and resilience during network or power outages.

Academically, the study will contribute empirical evidence at the barangay level by evaluating an AI-based flood risk classification model against traditional threshold-based methods in the Philippine context. It will assess performance using metrics such as accuracy, precision, recall, F1-score, false alarm rate, and lead time. The study will also demonstrate the integration of multi-source data, including LoRa-based sensor measurements and cached weather forecasts, within an offline-first architecture suitable for low-connectivity environments.

On a policy level, the findings will support the feasibility of barangay-scale flood early warning systems that can operate with minimal internet dependency. This aligns with national disaster risk reduction efforts and may inform future investments in scalable, edge-based monitoring systems for flood-prone communities.

Scope and Delimitations

This study will focus on the design and deployment of FLAIRE exclusively in Brgy. Elvita, Narra, Palawan, targeting a specific river section vulnerable to riverine flooding. It excludes coastal flooding, flash floods in upstream mountainous areas, and flood risks outside the barangay.

The system is an offline-first flood risk classification system that will provide a 24–48-hour predictive decision window using historical and real-time data. Historical data will be obtained from meteorological records of the Philippine Atmospheric, Geophysical and Astronomical Services Administration (PAGASA) and local flood occurrence records from the Barangay Disaster Risk Reduction and Management Committee (BDRRMC). In cases of limited localized data, preprocessing and feature engineering techniques will be applied to construct a usable dataset. All data will be stored in a local SQLite database for the Machine Learning model training and validation. Real-time data will be collected during the 2026 monsoon season using two LoRa-based sensor nodes equipped with ultrasonic water level sensors.

The system will integrate cached weather forecast data and real-time hydrological measurements on a local edge device, updated when internet connectivity is available. A Machine Learning model will be used to generate four standardized flood risk levels: Receding, Alert, Alarm, and Critical, and will be compared with traditional threshold-based warning methods.

Water level data and risk analysis will be accessible through a LAN-based web dashboard. The system will also support SMS notifications for key barangay officials, while final alert dissemination remains under the authority of the BDRRMC. Performance evaluation will use multi-class confusion matrix analysis and quantitative metrics such as accuracy, precision, recall, F1-score, false alarm rate, and lead time, along with qualitative feedback from BDRRMC staff, barangay officials, and selected residents.

This study is limited to a prototype deployment with two sensor nodes and does not provide full river system coverage. It relies on the availability and quality of historical and forecast data and does not guarantee prediction accuracy beyond the evaluated conditions.

Definition of Terms

Definitions of specialized terms, acronyms, or abbreviations used in the thesis

Artificial Intelligence (AI): In this study, refers to the use of a machine learning algorithm running on an edge device to analyze weather and sensor data for flood risk classification.

Cached Weather Data: Pre-downloaded weather forecasts (e.g., rainfall, precipitation probability, humidity, pressure) stored in a local SQLite database and updated when internet connectivity is available for offline use.

Confusion Matrix: A performance evaluation table used to compare predicted flood risk classifications against actual outcomes, serving as the basis for computing accuracy, precision, recall, and F1-score.

Edge Device: A local computing unit (e.g., Raspberry Pi) that receives sensor data, processes inputs using the Machine Learning model, stores data, hosts the dashboard, and manages SMS alerts.

False Alarm Rate: The proportion of predicted high-risk flood events (Alert, Alarm, Critical) that do not occur, used to assess system reliability.

Flood Risk Level: Categorical outputs (Receding, Alert, Alarm, Critical) generated by the model based on analyzed data, representing varying degrees of flood risk.

Hydrological Sensors: LoRa-enabled devices deployed along the river that measure real-time water levels using ultrasonic sensing.

LAN Web Dashboard: A locally hosted web interface accessible via LAN for monitoring real-time data, model outputs, and system status.

Lead Time: The time interval between the system’s risk alert and the actual occurrence of flooding, used to evaluate early warning capability.

Local Database: A SQLite database used to store historical data, weather forecasts, model data, and real-time sensor readings for offline operation.

LoRa (Long Range): A low-power wireless communication protocol used to transmit sensor data over long distances without internet connectivity.

BDRRMC (Barangay Disaster Risk Reduction and Management Committee): The local authority responsible for monitoring flood conditions and managing alert dissemination.

Model Training: The process of building the Machine Learning model using historical data to learn patterns associated with different flood risk levels.

Model Validation: The process of evaluating the trained model using separate data to assess its performance and generalization capability.

Multi-Channel Alert Dissemination: Delivery of alerts through multiple platforms, including the LAN dashboard and SMS notifications.

Offline-First Architecture: A system design approach where core functionalities operate without continuous internet connectivity, using local processing and storage.

Queued SMS: A mechanism where alert messages are stored locally and automatically sent once cellular connectivity becomes available.

Riverine Flooding: Flooding caused by the overflow of a river, which is the primary focus of this study.

Threshold-Based Warning System: A conventional method that triggers alerts when water levels exceed predefined limits.

Time-Series Data: Sequential data collected over time, such as water level measurements and weather variables, used as inputs for analysis.

Chapter 2

THEORETICAL FRAMEWORK

Review of Related Literature

Local

The National Disaster Risk Reduction and Management Council (National Disaster Risk Reduction and Management Council, 2020) adopted the National Disaster Risk Reduction and Management Plan (NDRRMP) 2020–2030 as the principal blueprint guiding national government agencies, local government units, civil society organizations, the private sector, and development partners on disaster and climate-resilient actions in the country. The plan establishes a Results-Based Monitoring and Evaluation System (RBMES) that defines 23 outcomes with measurable indicators, outputs, and activities to guide stakeholders in tracking and assessing contributions toward the country's disaster risk reduction vision. Among the significant challenges identified in the review of the previous NDRRMP 2011-2028 was the absence of established, functional, and standardized disaster risk reduction monitoring and evaluation systems at all levels of government. The NDRRMP 2020-2030 responds to these gaps by standardizing reporting tools and approaches, and by establishing a centralized digital platform for collecting, reporting, and sharing DRRM data. This national framework provides the policy context within which the development of community-based flood early warning systems, such as the one proposed in this study, is situated, and underscores the importance of technology integration and multi-level stakeholder participation in advancing disaster resilience in the Philippines.

A significant institutional development in Philippine flood management is the partnership between the Korea International Cooperation Agency (KOICA) and the Department of Science and Technology - Philippine Atmospheric, Geophysical and Astronomical Services Administration (DOST-PAGASA). In November 2023, the two agencies formalized their agreement for the "Development of Flood Forecasting and Early Warning System for the Greater Metro Manila and surrounding provinces of Laguna Lake Complex for Disaster Mitigation (EWS4) Project," with a grant aid of US$12.7 million covering a six-year implementation period from 2021 to 2026 (SERVANDO, 2023). The EWS4 Project involves the development of an advanced Flood Forecasting and Early Warning System (FFEWS), the establishment of hydrological observation and warning facilities, and the capacity building of local personnel. Key technologies deployed include x-band radar, rainfall and water level gauges, flow meters, and closed-circuit television systems strategically placed in key areas. Dr. Servando emphasized that the project is intended to increase the lead time for evacuation through more timely and accurate flood forecasts. This literature demonstrates that advanced, multi-sensor flood forecasting systems are already being institutionalized at the national level in the Philippines, lending credibility to the approach proposed in this study and highlighting the continued need to extend similar capabilities to rural, barangay-level communities.

As documented by Radyo Natin Nationwide (Radyo Natin Nationwide, 2020), the Provincial Government of Palawan, under the leadership of Governor Baham Mitra and the Provincial Disaster Risk Reduction Management Council, implemented a disaster risk-reducing initiative to install 20 Flood Early Warning Systems (FEWS) in flood-prone areas across the province. Significantly, Barangay Elvita in Narra was selected as the site for the first FEWS installation, establishing it as one of the officially identified flood-vulnerable communities in Palawan. According to the report, Barangay Elvita and adjacent barangays are regularly devastated by flash floods that inundate large tracts of agricultural lands for days, causing substantial economic disruption and placing lives and properties at risk. The installed FEWS consists of a solar-powered automatic water-level sensor designed with two-stage alert systems: an initial alert warns residents of rising water and the need to prepare for evacuation, while a second, louder alert triggers when floodwater reaches a critical level, signaling immediate evacuation. The device was modified from standard FEWS designs to suit the specific requirements of rural Palawan barangays and is mounted on a cement base with a steel frame in strategic community locations. This literature provides critical context for the selection of Brgy. Elvita as the study site, as it confirms the barangay's official recognition as a priority flood-vulnerable area by provincial authorities and documents the existing threshold-based warning infrastructure that the proposed AI-enhanced hybrid system aims to augment and improve upon.

The GSMA (GSMA, 2022) report on early warning systems in the Philippines highlights the country's extreme vulnerability to natural disasters, noting the devastating impact of events such as Typhoon Rai in December 2021, which was recorded as the second deadliest disaster globally that year. The report identifies mobile and digital technologies-including frontier technologies such as IoT, big data, and artificial intelligence-as playing an important role in implementing effective early warning systems that encompass the four key stages: risk knowledge, forecasting and warning, dissemination and communication, and preparedness and response. Although various initiatives have been piloted by government agencies, the private sector, and non-governmental organizations, the report notes that existing solutions in the Philippines are often fragmented and would benefit from higher-quality data and greater integration. The report calls for mobile network operators to build network resilience, for solution providers to design for local needs, and for government agencies to enhance risk knowledge and impact forecasting. These findings directly support the rationale for this study's development of an integrated, multi-channel, AI-based flood warning system designed specifically to address the local needs of Brgy. Elvita, Narra, Palawan.

The People in Need (PIN) and SMART Communications initiative known as STREAM-EWS, supported by the GSMA Innovation Fund, demonstrates the practical effectiveness of combining IoT infrastructure with mobile technology for community-based early warning in the Philippines (Jenkins, 2025). The project resulted in more than 55,000 direct users and 335,000 indirect community members receiving critical alerts, supported by the installation of 32 weather stations feeding into early warning systems. Over 1,500 disaster risk reduction stakeholders were engaged, including emergency responders and civil society actors. The initiative underscores the scalability and community impact achievable through cross-sector partnerships that integrate private sector innovation with local community engagement. Notably, the project demonstrated that localized early warning messages can reach the most vulnerable populations when technological platforms are combined with grassroots disaster risk reduction participation. This case study supports the design philosophy of the proposed system, which similarly aims to deliver localized, actionable flood alerts to barangay-level communities through multiple communication channels.

The 2024 Annual Report of the Department of Science and Technology – Advanced Science and Technology Institute (Department of Science and Technology - Advanced Science and Technology Institute, 2024), themed "Together, We Can," highlights the agency's ongoing investments in AI, IoT, microelectronics, and ICT for national development and disaster resilience. Key projects include the FLASH (weather forecasting using lightning data) and SAGAP (geospatial signal assessment) programs, which are central to the agency's goal of building a "greener, smarter, and more resilient Philippines." The report also describes the launch of the DIMER (Decentralized Intelligent Model Exchange Repository), a "model store" designed to democratize AI by providing ready-to-use machine learning models. The agency's active investment in AI and IoT applications for disaster risk reduction reflects the national direction toward integrating intelligent, connected technologies into hazard monitoring and early warning systems. This literature affirms that AI-based approaches to disaster warning are consistent with the current strategic priorities of Philippine science and technology institutions, providing institutional support for the AI-driven methodology adopted in this study.

Foreign

Gray (Gray, 2023) of the International Telecommunication Union (ITU) provides a state-of-the-art review of mobile-based early warning systems, noting that by 2022, ninety-five percent of the world's population had access to mobile broadband networks and three-quarters of people owned a mobile phone, making mobile networks a powerful communication channel for hazard alerts. The report describes two complementary technologies, location-based SMS and cell broadcast, both of which can deliver targeted, geo-located warnings without network congestion. The report recommends combining multiple communication technologies and channels, including sirens, radio, TV, mobile alerting apps, and social media, to ensure coherent and redundant messaging. It cites the European Union's regulatory mandate requiring member states to implement mobile-based early warning systems as an example of how regulatory frameworks can accelerate widespread adoption. The UN Early Warnings for All (EW4A) Initiative, launched in March 2022, similarly calls for universal protection of every person by an early warning system by 2027, with geo-located mobile-based services as a critical element. These international standards and recommendations validate the multi-channel alert dissemination architecture of the proposed system, which incorporates SMS notifications and a web dashboard to achieve redundancy and maximize community reach.

Budimir et al. (Budimir, et al., 2025) present a critical examination of the challenges and opportunities in implementing people-centered multi-hazard early warning systems (MHEWS) in the Global South. Drawing on extensive experience from Practical Action, the study highlights a significant mismatch between global ambitions, such as those embodied in the Early Warnings for All initiative, and the operational realities on the ground in vulnerable communities. The authors argue that multi-hazard thinking must integrate hazard interrelationships, vulnerability dynamics, and the complexity of multi-hazard impacts, and emphasize the need for localized, inclusive strategies that genuinely address the needs of the most vulnerable populations. The study identifies that current MHEWS implementations often fail to reach those at greatest risk due to inadequate community engagement, technology gaps, and insufficient localization of warning messages. These findings reinforce the importance of designing the proposed system with community-level deployment, local stakeholder engagement, and context-specific alert thresholds that reflect the particular hydrological and social conditions of Brgy. Elvita, Narra, Palawan.

Semtech (Semtech Corporation, 2024) provides a definitive technical description of LoRa (Long Range) RF modulation technology and the LoRaWAN open protocol, which together constitute the primary wireless communication standard for low-power wide area networks (LPWANs). LoRa enables data transmission over distances of up to 10 miles (15 kilometers) or more in rural line-of-sight conditions, and up to 3 miles (5 kilometers) in dense urban environments. A key characteristic of LoRa-based solutions is ultra-low power consumption, which allows battery-operated devices to remain operational for up to 10 years. A single eight-channel LoRaWAN gateway can support hundreds of thousands of messages over a 24-hour period, capable of serving up to 10,000 devices transmitting 10 messages per day. The network architecture follows a star topology, minimizing the number of gateways required to serve a large number of end nodes while keeping capital and operational costs low. These technical characteristics make LoRa-based sensor networks ideally suited for remote hydrological monitoring in rural barangays like Brgy. Elvita, where long communication distances, limited power infrastructure, and the need for cost-effective deployment are primary design constraints.

Review of Related Studies

Local

Cacal, Austria, and Taboada (Cacal, Austria, & Taboada, 2023) conducted a rainfall-runoff simulation study for the Irawan basin in Puerto Princesa City, Palawan, Philippines, a geographic context directly relevant to the proposed study. Using ArcGIS and QGIS for geospatial analysis, and the HEC-HMS hydrological model incorporating SCS loss, Clark unit hydrograph, and Muskingum flow routing, the researchers simulated a heavy rainfall event that occurred on December 18, 2017, in the ungauged basin to determine peak flow volumes. Hydrological parameters were determined using soil type, land use, and land cover maps, with model performance evaluated using Nash-Sutcliffe efficiency (NSE) and Percentage Bias (PBIAS) metrics. The study concludes that an effective decision-making and warning system can be implemented using the developed model and emphasizes the role of GIS-based simulation in supporting flood risk assessment for data-scarce Philippine basins. While the proposed study does not employ physics-based hydrological simulation, the Cacal et al. (2023) study underscores the critical importance of local watershed data and geospatial context in flood prediction within the Palawan setting, and establishes that data-driven approaches can be applied to flood-prone river systems in the same region as this study.

Kim et al. (Kim, et al., 2023) conducted a directly relevant study that utilized deep learning models to forecast flood water levels in the CarayCaray Basin on Biliran Island in Eastern Visayas, Philippines, and applied flood risk classification to evaluate the degree of risk associated with predicted water levels. The study employed deep neural network (DNN) models for water level prediction, achieving an NRMSE of 9.48, with the DNN model demonstrating the best predictive performance. For flood risk classification, the researchers utilized extreme gradient boosting, random forest, and decision tree models, with the extreme gradient boosting model achieving the highest F1-score of 0.92. The study emphasizes that AI-based predictive models are anticipated to reduce flood damage and enhance disaster mitigation systems, and that real-time collection of rainfall and water level data enables continuous model learning. The study's application of AI models within the Philippine flood context, combined with its demonstration that random forest and gradient boosting algorithms can achieve high classification accuracy for flood risk levels, provides strong empirical support for the AI-based classification approach adopted in the proposed study.

Macalalad et al. (Macalalad, et al., 2021) addressed the challenge of flash flood modeling in data-poor river basins, using the Matina River basin in Davao City as a case study. The researchers employed the Liuxihe physically-based distributed hydrological model (PBDHM), which can directly determine model parameters from remotely-sensed underlying surface characteristics, enabling flood forecasting in areas with little to no in-situ observation data. The study found good correspondence between past heavy rainfall events and simulated river discharges that conform to hydrological regularities, establishing the Liuxihe model as a viable baseline for developing flood forecasting systems in the rivers of Davao City. This study is significant to the proposed research because it highlights the common challenge of data scarcity in Philippine river basins and demonstrates that hybrid approaches combining remote sensing data with hydrological modeling can overcome this limitation. The proposed study similarly addresses data availability challenges through the integration of real-time LoRa sensor data with weather forecast API inputs, providing a complementary machine-learning-based solution to the data-poor modeling challenge documented by Macalalad et al. (2021).

Miyamoto et al. (Miyamoto, et al., 2022) developed a comprehensive methodology for enhancing flood resilience in Davao City through a co-design approach involving multiple levels of stakeholders. The study implemented the Online Synthesis System for Sustainability and Resilience, which integrates different scientific disciplines, and fostered Facilitators to bridge the science community and society. Basin- and barangay-scale hydrological models enabled real-time flood forecasting and climate change impact assessment, while co-designed e-learning workshops trained approximately thirty Facilitators who developed twenty-one risk communication plans for fourteen barangays. The study demonstrates that the synthesis of science-based knowledge and human resource development can fill the gaps between the scientific community and society in flood resilience efforts, and emphasizes that risk communication must be tailored to the geographic, demographic, economic, and social features of individual barangays. This study reinforces the barangay-level, stakeholder-engaged design approach adopted in the proposed study, and validates the importance of developing alert systems that can be directly operated and maintained by local disaster risk reduction personnel with the technical capacities appropriate to their community context.

Mesias and Bagtasa (Mesias & Bagtasa, 2025) developed an AI-based tropical cyclone rainfall forecast model for the Philippines using machine learning, specifically combining a self-organizing map (SOM) for clustering TC tracks with a random forest (RF) regression model that predicts accumulated TC rainfall from parameters including TC position, intensity, translational speed, and locale-specific characteristics. The RF model determined that distance to the tropical cyclone center has the most influence on accumulated rainfall variability, followed by TC duration, latitude of land grid, and TC track type. The model produced rainfall distributions similar to calibrated satellite rainfall observations and demonstrated particular skill in predicting intense rainfall events compared to other statistical or dynamical weather models, including the WRF model. The study concludes that the RF model's predictive ability, combined with its low computational power requirements, makes it a potential tool for augmenting TC rainfall forecasting in the Philippines. The application of Random Forest to Philippine rainfall and hazard prediction in Mesias and Bagtasa (2025) supports Random Forest as a viable classification algorithm option for the proposed flood risk classification system, demonstrating its suitability and effectiveness within the Philippine meteorological context.

Foreign

Ahmed (Ahmed, 2021) designed and developed a low-powered IoT-based sensor system for real-time monitoring of water levels inside underground drainage systems, presenting literature reviews of water level sensing technologies and LPWAN technologies, including a feasibility analysis comparing LoRa and NB-IoT for manhole communication. The developed prototype demonstrated water level measurement capability up to 1 meter with ±2 cm accuracy, consuming 69.2 µA average current and capable of operating for more than 6.5 years on a 4,000 mAh battery. The study evaluates hardware setup, software development, and cloud infrastructure implementation for the prototype, with results confirming that LoRa-based solutions offer a technically viable and energy-efficient platform for real-time water level monitoring. While Ahmed's study focuses on urban underground drainage, the core design principles, low-power sensor design, LoRa communication feasibility, cloud data integration, and sensor accuracy requirements—are directly transferable to the proposed study's deployment of LoRa-based hydrological sensors along riverbanks in rural Brgy. Elvita. This study validates the technical feasibility of the proposed sensor network architecture and informs the hardware specifications for the study's LoRa sensor node design.

Schumacher et al. (Schumacher, et al., 2021) document the development and operational transition of the Colorado State University Machine Learning Probabilities (CSU-MLP) system, a probabilistic forecast system for excessive rainfall events in the contiguous United States. The system is based on ensemble reforecasts, precipitation observations, and machine learning algorithms, specifically random forests, and was designed to generate Excessive Rainfall Outlooks (EROs) as a decision-support tool for Weather Prediction Center forecasters at lead times of 1-3 days. Through iterative improvements resulting from close collaboration between CSU and WPC and evaluation at the Flash Flood and Intense Rainfall Experiment, the CSU-MLP forecasts were found to be skillful and reliable and were successfully transitioned into operational use. The study represents a documented example of a successful research-to-operations transition for a random forest–based flood forecasting system and highlights the potential for machine learning and post-processing techniques to improve operational rainfall and flood predictions. It supports Random Forest as a viable candidate model for flood risk prediction in the proposed study and demonstrates that machine learning–based probabilistic forecasting systems can be systematically validated and integrated into real-world forecasting workflows.

Sehgal, Gurudiwan, and Pant (Sehgal, Gurudiwan, & Pant, 2025) proposed a scalable framework for real-time flood prediction that integrates remote sensing data, collected via satellites and drones, with edge computing technologies to achieve high-precision, low-latency disaster management. The framework involves retrieving required remote sensing data, processing it, and running machine learning models on edge devices to produce rapid warning signals. Results demonstrated a reduction in latency and improved data accuracy compared to conventional cloud-only processing approaches, enabling quicker warning signals to be generated and disseminated. The study underscores the importance of integrating emerging technologies, remote sensing, machine learning, and edge computing, to improve resilience and responsiveness of humanitarian aid systems in disaster regions. While the proposed study does not employ satellite remote sensing or edge computing, Sehgal et al.'s framework emphasizes the critical role of low-latency, real-time data processing pipelines in effective flood prediction, and demonstrates that integrating multiple data acquisition technologies with machine learning models can substantially improve warning timeliness, a principle that motivates the proposed system's integration of real-time LoRa sensor data with weather forecast API inputs.

Refadah (Refadah, 2025) presents a comprehensive review of machine learning (ML) and artificial intelligence (AI) models applied to flood prediction, benchmarking various ML algorithms for their performance in short- and long-term flood forecasting scenarios. The review assesses multiple ML algorithms, including ensemble methods, deep learning architectures, and hybrid models, in terms of resilience, accuracy, efficacy, and computational speed. Key trends identified include hybridization of models, data decomposition techniques, and model ensemble methods using satellite-based impact analysis, all of which are found to increase the precision and dependability of flood prediction systems. The review highlights that in the past twenty years, ML models have become more effective at creating precise and cost-effective prediction systems, and that these models have significant potential to minimize flood damage and loss of human life. Refadah's systematic review provides a comprehensive empirical basis for the selection of machine learning as the core predictive methodology in the proposed study, and specifically supports the use of ensemble methods, such as Random Forest, which consistently demonstrate strong performance across diverse flood forecasting scenarios in the reviewed literature.

Bibi et al. (Bibi, Saleem, Jalil, Nasir, & Alsubait, 2025) investigated riverine flood prediction and early warning for the Kabul River transboundary basin between Pakistan and Afghanistan, a context characterized by complex mountainous terrain, extreme climate variability, and significant challenges in obtaining upstream data, which are common barriers to effective flood forecasting in similar global river basins. The study applied and compared multiple advanced machine learning and deep learning models, including Support Vector Machines (SVM), XGBoost, Artificial Neural Networks (ANN), Long Short-Term Memory (LSTM) networks, and Gated Recurrent Units (GRU), for predicting daily and multi-step river flow using satellite-based climatic data. The LSTM network outperformed all other models, achieving an R² of 0.96 and an RMSE of 140.96 m³/s, with time-series LSTM and GRU models performing well for short-term forecasts of up to five days. However, accuracy declined beyond the fourth day, underscoring the need for longer historical datasets for reliable long-term prediction. The study's findings on the comparative performance of AI models for riverine flood prediction in data-limited transboundary basins are directly applicable to the proposed system, which similarly addresses flood prediction challenges in a data-scarce Philippine river basin. The study confirms that multi-step AI-based riverine flood risk classification is technically achievable within 24-48-hour windows, the prediction horizon targeted by the proposed system, and supports the selection of ensemble and tree-based methods as strong candidates for operational deployment where computational efficiency is a design priority.

Conceptual Framework

This study will be guided by a conceptual framework that will illustrate the relationships among key inputs, system processes, and outputs in the design and development of FLAIRE, an offline-first AI-based riverine flood risk classification and multi-channel early warning system for Brgy. Elvita, Narra, Palawan.

The framework will consist of three primary input components. The first will include cached weather forecast data obtained from public APIs and updated when internet connectivity is available, including rainfall, precipitation probability, humidity, and atmospheric pressure. The second will consist of real-time hydrological data collected from two LoRa-based sensor nodes, including ultrasonic water level measurements and derived features such as rate-of-rise. The third will include historical reference data obtained from meteorological records and local flood occurrence data, which will be used for model training and validation.

These inputs will be integrated and processed by a Machine Learning classification model to be deployed on a local edge device. The model will analyze combined data inputs along with engineered temporal features, such as water level trends and recent changes, to generate categorized flood risk levels: Receding, Alert, Alarm, and Critical.

The resulting classifications will serve as the system outputs, which will be delivered through a multi-channel alert mechanism. This will include a LAN-accessible web dashboard for monitoring and queued SMS notifications for dissemination to barangay officials, ensuring functionality even during network disruptions.

The framework is expected to support improved situational awareness and decision-making by providing a 24–48-hour predictive decision window. However, the system will function as a decision-support tool, with final alert actions remaining under the authority of the Barangay Disaster Risk Reduction and Management Committee (BDRRMC).

Research Paradigm

This study will follow a developmental and evaluative research paradigm grounded in the post-positivist tradition, employing an Input-Process-Output (IPO) model to guide the design, development, and evaluation of the proposed hybrid AI-based flood risk prediction and early warning system. The paradigm maps the logical flow of variables, processes, and outcomes that constitute the research investigation.

Figure 2.1

Schematic presentation of conceptual framework

Feedback

Weather Forecast API Data, LoRa-Based Hydrological Sensor Data, Historical Reference Data

Data Acquisition and Preprocessing, Feature Engineering, Model Development and Training, Baseline Comparison, System Integration

FLAIRE: AI Offline-First Flood Early Warning System Using LoRa Sensors for Brgy. Elvita, Narra, Palawan

Figure 2.1 establishes a clear directional relationship in which the independent variables (weather forecast and sensor inputs, historical data) are transformed through defined computational and analytical processes into the dependent outcomes (flood risk classification, alert delivery, and system performance measures). This structure ensures that the study's research questions, objectives, and evaluation criteria are systematically aligned, and that the contributions of the system can be rigorously and transparently assessed within a scientific framework.

Chapter 3

RESEARCH METHODOLOGY

Research Design

This study will employ a Research and Development (R&D) design following the four-phase Design Research Methodology (DRM) framework proposed by Blessing and Chakrabarti (2009), consisting of Research Clarification, Descriptive Study I, Prescriptive Study, and Descriptive Study II. These phases will be adapted into planning and design, system development, field testing, and evaluation for the development of FLAIRE.

Figure 3.1

Planning & Design

System Development

Field Testing

Evaluation

DRM Framework

Figure 3.1 illustrates the Research and Development (R&D) process that will be used as a guide to the development of the system. The R&D approach allows iterative refinement of the system, including the Machine Learning model, dataset preprocessing, and overall architecture, based on results obtained during testing and field deployment. This flexibility is essential given the evolving nature of data-driven systems and the variability of environmental conditions in Brgy. Elvita.

The study will incorporate descriptive and evaluative methods to assess both user acceptability and system performance. Quantitative data will be collected through structured survey instruments administered to target respondents, while system performance will be evaluated using standard machine learning metrics such as accuracy, precision, recall, F1-score, false alarm rate, and lead time.

The Machine Learning flood risk classification model will be compared with a traditional threshold-based warning method using the same dataset and evaluation metrics to ensure consistency. This comparison aims to determine differences in performance and practical applicability within the barangay context.

Locale of the Study

The study will be conducted in Brgy. Elvita, Narra, Palawan, Philippines. The barangay is selected due to its documented exposure to recurrent riverine flooding, proximity to a major river system, and its relevance as a deployment site for localized flood early warning solutions. (Radyo Natin Nationwide, 2020).

FLAIRE's LoRa-based hydrological sensor nodes (2 units) will be installed along a selected river section. Sensor placement will be determined through site assessment and consultation with the Barangay Disaster Risk Reduction and Management Committee (BDRRMC) and local stakeholders to ensure adequate coverage of flood-prone areas.

Figure 3.2

Locale of the Study

Brgy. Elvita

Flood-Prone area

Narra

Figure 3.3 shows the location situated near the flood-prone area of Brgy. Elvita, Narra, Palawan.

Flowchart of the Proposed System

The flowchart outlines the step-by-step process involved in the device operation, ensuring a clear and logical sequence of activities.

Figure 3.3.1

LoRa-enabled Hydrological sensor nodes flowchart

Start

Collect

Sensor Data

Transmit to

Edge Device via LoRa and SMS

End

Turn off device?

Yes

No

Figure 3.3.1 illustrates the operational workflow of the LoRa-enabled hydrological sensor node. The primary function of this node is to collect data on the river’s water level and transmit it to the edge device for processing every 10 minutes.

Figure 3.3.2

Edge device flowchart

Start

Collect Weather Forecast Data via API

Is Sensor Data available?

Wait for Data transmission

Data Processing:

- Cleaning

- Synchronization

Store Data to

Database

A

No

Yes

B

End

Parallel Risk Assessments

Finalize Flood

Risk Assessment

A

Threshold-based

Risk Logic

Machine Learning Classification

Log results

Turn off device

B

Yes

No

Update Dashboard

Figure 3.3.2 provides a visual representation of the Edge Device’s operational workflow. The Edge Device collects cached weather forecasts and real-time water-level readings, then processes these inputs using threshold logic and a Machine Learning classifier to determine flood-risk level.

Figure 3.3.3

LAN Dashboard

Start

Real-time Data

End

Close?

Yes

No

Show Dashboard

Access Edge Device via WEB BROWSER

(192.168.x.x)

Display Data Graphs

Display Risk

Risk Assessment

History

Display Logs

Alert Trigger

Send SMS

Log action

Figure 3.3.3 shows the workflow for the LAN-accessible dashboard available to BDRRMC officials. It serves as an offline-capable access point for viewing real-time data, edge-analyzed risk assessments, and logs. The dashboard also offers the option to trigger alerts, which are managed by BDRRMC officials.

Target Respondents

The target respondents will evaluate FLAIRE in terms of acceptability, functionality, usability, reliability, and perceived contribution to disaster preparedness. Respondents will be selected using purposive sampling based on their roles and relevance to the system’s deployment context.

Participants will include Barangay Disaster Risk Reduction and Management Committee (BDRRMC) personnel, who will serve as primary system users through the LAN-based dashboard for monitoring and decision support; barangay officials responsible for local disaster coordination; and selected community representatives from flood-affected households as secondary stakeholders providing feedback.

The sample size will be determined based on stakeholder availability and relevance to ensure adequate representation across user groups for meaningful evaluation.

Instrumentation

Data for this study will be collected using several instruments and tools designed for FLAIRE’s offline-first architecture. A structured survey questionnaire based on the Technology Acceptance Model (TAM) will be used to assess system acceptability in terms of functionality, usability, reliability, and perceived contribution to disaster preparedness. The instrument will use a 5-point Likert scale (1 = Strongly Disagree to 5 = Strongly Agree) and will undergo content validation by subject matter experts and pilot testing prior to deployment.

Hardware instrumentation will include two LoRa-based sensor nodes equipped with ultrasonic water level sensors integrated with ESP32 microcontrollers. A Raspberry Pi-based gateway will receive sensor transmissions and store data in a local SQLite database hosted on the edge device.

The software stack will include a cached weather data pipeline that stores forecast data obtained from the OpenWeatherMap API when internet connectivity is available, a Machine Learning classification model implemented using the scikit-learn library running locally on the edge device, and a Flask-based LAN web dashboard for BDRRMC monitoring. A GSM modem will be used to support queued SMS alert delivery.

System performance will be evaluated using multi-class confusion matrix analysis to compare predicted and actual flood risk classifications, from which accuracy, precision, recall, and F1-score will be derived. Additional metrics such as false alarm rate and lead time will also be used. The Machine Learning model outputs will be compared with a threshold-based warning method using the same dataset to ensure a consistent and fair evaluation.

Gantt Chart

This section outlines the device's development timeline, covering phases from planning to finalization. It details each stage, ensuring timely completion and smooth progression, contributing to the successful development and efficient execution of the project.

Figure 3.4

Duration of the development of the system

|  | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Planning & Research |  |  |  | Feb; Mar; Apr; May; Jun; Jul; Aug; Sep; Oct; Nov; Dec. Feb; Mar; Apr; May; Jun; Jul; Aug; Sep; Oct; Nov; Dec. |  |  |  |  |
| Design |  |  |  |  |  |  |  |  |
| Development |  |  |  |  |  |  |  |  |
| Test |  |  |  |  |  |  |  |  |
| Finalizing |  |  |  |  |  |  |  |  |

Legend:

Duration

Start Task

End Task

The planning phase started in January and will continue until March, followed by the analysis phase, which will span from early April and extends through May. Each task was marked by solid bars, with arrows indicating the start and end points. This structured timeline helps visualize the project`s progress, highlight dependencies, and ensure that each phase transitions smoothly into the next, ultimately aiding in effective project management

Cost and Benefits Analysis

This section estimates expected costs for implementation and discusses benefits to the barangay and stakeholders. Final costs are dependent on supplier pricing, Palawan logistics, and deployment of 2 sensor nodes. The budget prioritizes local edge computing and renewable power suitable for rural deployment.

Table 3.1

Material and Cost of System

| Material | Cost |
| --- | --- |
| WiFi LoRa 32 V4 ESP32 BT development board x3 | ₱ 5,400 |
| Raspberry Pi AI HAT+ 2 | ₱ 9,000 |
| Ultrasonic Water Level Sensor x2 | ₱ 800 |
| Enclosure / Weatherproof Housing x2 | ₱ 2,000 |
| Power Supply / Solar Panel Kit x2 | ₱ 3,000 |
| Miscellaneous Hardware | ₱ 2,000 |
| Weather Forecast API | ₱ 2,000 |
| SMS Gateway Service (yearly) | ₱ 2,000 |
| Transportation and field deployment | ₱ 1,000 |
| Printing, documentation, and miscellaneous | ₱ 1,000 |
| Total | ₱28,200 |

Table 3.1 details materials for FLAIRE's offline-first deployment. ESP32 LoRa V3 boards serve as sensor nodes transmitting water level via LoRa to the Raspberry Pi 4 edge device, which hosts the local SQLite database, Machine Learning model, and Flask LAN dashboard. Solar panel kits ensure continuous operation during typhoon season power outages. Cached weather data and GSM modem (queued SMS delivery) eliminate cloud dependency. The IP67 enclosures protect against Palawan monsoon conditions. This cost-effective design demonstrates scalability for other barangays at approximately ₱9,400 per sensor node

Benefits Analysis:

FLAIRE is expected to provide technical, operational, and social benefits tailored to the rural context of Brgy. Elvita. The system will support a 24–48-hour predictive decision window through local AI analysis of cached weather data and real-time LoRa sensor inputs, enabling more informed decision-making by the BDRRMC via a LAN-accessible web dashboard.

The system incorporates redundant, offline-capable alert mechanisms, including a local dashboard and queued SMS delivery through a GSM modem, reducing reliance on continuous internet connectivity and minimizing the risk of communication failure during extreme weather events. From a technical perspective, the study will contribute to the development of a localized flood dataset stored in SQLite format, support empirical comparison between a Machine Learning classification model and a threshold-based warning method using standard evaluation metrics, and demonstrate a scalable prototype suitable for deployment in similar flood-prone barangays.

In terms of social impact, the system is expected to enhance community preparedness by providing timely and accessible flood risk information, supporting early response actions, and improving overall disaster awareness among stakeholders.

System Maintenance Plan

Maintenance procedures are proposed to support the reliability and continuous operation of FLAIRE in Brgy. Elvita. Routine monitoring will be conducted through the edge device dashboard to track sensor uptime, signal strength, battery levels, database status, and model performance. Periodic on-site inspections will be performed every three months to verify enclosure integrity, clean solar panels, test GSM connectivity, and recalibrate ultrasonic sensors.

Corrective maintenance will address hardware failures, physical damage, and software issues through component replacement, system resets, and database recovery when necessary. Adaptive maintenance will include periodic retraining of the Machine Learning classification model using newly collected data and updates to system parameters based on observed conditions and stakeholder feedback.

Data security and backup measures will include automated local database backups, offsite storage, and role-based access control within the LAN environment. Maintenance responsibilities will be shared between designated barangay personnel and system developers to ensure sustainability of the deployed system.

Developer and Client Communication

A structured communication plan is proposed to support coordination between the researcher, Brgy. Elvita officials, and BDRRMC personnel throughout the development and deployment of FLAIRE. This will include regular coordination meetings, progress updates, and shared documentation to facilitate requirements validation, system deployment, and user training.

Issue reporting and resolution will follow a structured process, where concerns are documented, classified by severity, and addressed through appropriate technical actions. Relevant documentation, including user manuals, troubleshooting guides, and system architecture references, will be provided to support system use and sustainability.

Chapter 4

RESULTS AND DISCUSSION

Chapter 5

SUMMARY OF FINDINGS, CONCLUSIONS, AND RECOMMENDATIONS

References

Ahmed, H. (2021). Low Power IoT Sensor System for Underground Drainage Monitoring. Tallinn University of Technology. Retrieved from https://digikogu.taltech.ee/et/Download/b456bb87-faa4-4b32-adc6-757bb5e79321

Bibi, H., Saleem, S., Jalil, Z., Nasir, M., & Alsubait, T. (2025). Riverine Flood Prediction and Early Warning in Mountainous Regions using Artificial Intelligence. arXiv, 18645. Retrieved from doi.org/10.48550/arXiv.2505.18645

Blessing, L. T., & Chakrabarti, A. (2009). DRM, a Design Research Methodology. Springer-Verlag London Limited. Retrieved from https://content.e-bookshelf.de/media/reading/L-8059-9cd0399cfa.pdf

Budimir, M., Trogrlić, R. Š., Almeida, C., Arestegui, M., Vásquez, O. C., Cisneros, A., . . . Ndiaye, A. (2025). Opportunities and challenges for people-centered multi-hazard early warning systems: Perspectives from the Global South. iScience, 112353. doi:10.1016/j.isci.2025.112353

Cacal, J. C., Austria, V. C., & Taboada, E. B. (2023). Extreme Event-based Rainfall-Runoff Simulation Utilizing GIS Techniques in Irawan Watershed, Palawan, Philippines. Civil Engineering Jounal, 220–232. Retrieved from https://doi.org/10.28991/CEJ-2023-09-01-017

Department of Science and Technology - Advanced Science and Technology Institute. (2024). 2024 DOST-ASTI Annual Report. DOST-ASTI. Retrieved from https://asti.dost.gov.ph/wp-content/uploads/2024-DOST-ASTI-Annual-Report.pdf

Gray, V. (2023). Mobile-based Early Warning Systems: For effective and inclusive alerting. Global Disaster Preparedness Center / IFRC. Retrieved from https://preparecenter.org/wp-content/uploads/2023/04/briefing_note_mobile-based-early-warning-systems.pdf

GSMA. (2022). Early Warning Systems in the Philippines: Building resilience through mobile and digital technologies. GSMA. Retrieved from https://www.gsma.com/solutions-and-impact/connectivity-for-good/mobile-for-development/wp-content/uploads/2022/06/PhilippinesEWS_R_Web.pdf

Jenkins, T. (2025, September 2). Case Study (Philippines): IoT and Mobile Tech for Anticipatory Action. Retrieved from People in Need Philippines: https://philippines.peopleinneed.net/en/case-study-philippines-iot-and-mobile-tech-for-anticipatory-action-12880gp

Kim, D., Park, J., Han, H., Lee, H., Kim, H. S., & Kim, S. (2023). Application of AI-Based Models for Flood Water Level Forecasting and Flood Risk Classification. KSCE Journal of Civil Engineering, Volume 27, Issue 7, Pages 3163-3174. Retrieved from linkinghub.elsevier.com/retrieve/pii/S1226798824040157

Macalalad, R. V., Xu, S., Badilla, R. A., Paat, S. F., Tajones, B. C., Chen, Y., & Bagtasa, G. (2021). Flash flood modeling in the data-poor basin: A case study in Matina River Basin. Tropical Cyclone Research and Review, 87-95. Retrieved from https://doi.org/10.1016/j.tcrr.2021.06.003

Mesias, C. G., & Bagtasa, G. (2025). AI-Based Tropical Cyclone Rainfall Forecasting in the Philippines Using Machine Learning. Meteorological Applications, Volume 32, Issue 4, e70083. Retrieved from https://doi.org/10.1002/met.70083

Miyamoto, M., Kakinuma, D., Ushiyama, T., Rasmy, A. W., Yasukawa, M., Bacaltos, D. G., . . . Kitsuregawa, M. (2022). Co-Design for Enhancing Flood Resilience in Davao City, Philippines. Water, 978. Retrieved from https://www.mdpi.com/2073-4441/14/6/978

National Disaster Risk Reduction and Management Council. (2020). National Disaster Risk Reduction and Management Plan 2020 - 2030: RESULTS-BASED MONITORING AND EVALUATION PLAN. NDRRMC. Retrieved from https://rbmes-api.ndrrmc.gov.ph/download/RBMES_NDRRMP_2020-2030.pdf

Radyo Natin Nationwide. (2020, October 28). Palawan installs 20 flood early warning devices. Retrieved from Radyo Natin Nationwide: https://www.radyonatin.com/post.php/2114.ea5155cc7ab189f9

Refadah, S. S. (2025). Development in flood forecasting: A comprehensive review of complex and machine learning models. Physics and Chemistry of the Earth, Parts A/B/C, Volume 139, 103975. Retrieved from https://doi.org/10.1016/j.pce.2025.103975

Schumacher, R. S., Hill, A. J., Nelson, J. A., Erickson, M. J., Trojniak, S. M., & Herman, G. R. (2021). From Random Forests to Flood Forecasts: A Research to Operations Success Story. Bulletin of the American Meteorological Society, Vol. 109, Issue 9, E1742–E1755. Retrieved from https://doi.org/10.1175/BAMS-D-20-0186.1

Sehgal, D. R., Gurudiwan, P., & Pant, S. (2025). Real-Time Flood Prediction Using Remote Sensing and Edge Computing. International Journal of Environmenal Sciences, 1459. Retrieved from https://doi.org/10.64252/7qha5q27

Semtech Corporation. (2024, March). LoRa® and LoRaWAN®. Semtech Corporation. Retrieved from Semtech Corporation: https://www.semtech.com/uploads/technology/LoRa/lora-and-lorawan.pdf

SERVANDO, N. T. (2023, November 21). KOICA and DOST-PAGASA Sign Agreement to Enhance Disaster Resilience. Retrieved from PAGASA: https://www.pagasa.dost.gov.ph/press-release/146

BIONOTES

UNDERGRADUATE RESEARCHER

| Nathaniel S. Betitta | Nathaniel S. Betita was born on December 26, 2000. He is a third-year student at Palawan State University Narra-Campus, where he is currently pursuing a Bachelor of Science in Computer Science. He resides in Barangay Panacan II, Narra, Palawan. With a keen interest in technology and innovation, he aspires to pursue a career as a game developer. |
| --- | --- |

THESIS/DISSERTATION ADVISER

| Joel P. Gulane Jr. M.I.T. | Joel P. Gulane Jr. is a dedicated educator and the current department of the Computer Science program at Palawan State University Narra Campus. His commitment to education is evident in his passion for teaching, training, and his involvement in the technology and computer-related industries. |
| --- | --- |

Appendices

Appendix A

USER MANUAL

Appendix B

SOURCE CODE

Appendix C

SURVEY QUESTIONAIRE

Appendix D

INTERVIEW, PROTOTYPE, DOCUMENTATION

Appendix E

LETTERS AND OTHER COMMUNICATIONS

Appendix F

GRAMMARIAN CERTIFICATION

Appendix G

|  |  | NATHANIEL S. BETITA Brgy. PANACAN II, Narra, Palawan Mobile #: 0994-017-6150 |
| --- | --- | --- |
| PERSONAL DATA: |  |  |
| Age: | 25 |  |
| Sex: | Male |  |
| Birthday: | December 26, 2000 |  |
| Height: | 5`5 |  |
| Weight | 55 |  |
| Religion: | The Church of Jesus Christ of Latter-day Saints |  |
| Nationality: | Filipino |  |
| Parents: | William T. Betita Arlene S. Betita |  |
| EDUCATIONAL BACKGROUND: |  |  |
| Tertiary: | Palawan State University NIA-MBBRIS Compound, Narra, Palawan Bachelor of Science in Computer Science |  |
| Senior High: | Narra National High School Narra, Palawan |  |
| Junior High: | Narra National High School Narra, Palawan |  |
| Elementary: | Narra Pilot School Narra, Palawan |  |

CURRICULUM VITAE

P A L A W A N   S T A T E   U N  I V E R S I T Y – N A R R A   C A M P U S